Interface Control Document (ICD)

- Scope

SEDAP-Express is an exceptionally fast path to integrate new applications, sensors, effectors or other similar things into the ecosystem of UNIITY, but not only! There are already a bunch of companies and individuals who use that internally for their own projects to exchange tactical or telemetry data. That's why it is intentionally kept simple and offers several technical ways of communication. Of course, this results in limitations, but in most cases where quick and easy integration is required, these are negligible. If increased demands arise later on, the “bigger” SEDAP API respective UNIITY interface can be used if necessary. SEDAP-Express is licensed under the “Simplified BSD License” (BSD-2-Clause). Therefore, using SEDAP-Express in commercial or non-commercial projects should be straightforward. Everything you need for development and testing can be found here on the Internet at https://SEDAP.Express. It is also available in the most common repositories.

- Glossary

UNIITY		=	Unified, Networked Integration of Innovative TechnologY

SEDAP		=	Safety critical Environment for Data exchange And Process scheduling

CSV			=	Comma-Separated-Values

SEC			=	SEDAP-Express-Connector (Connector which is a part of UNIITY)

MockUp/TestTool	=	Simulation of the real SEC and a C2-like system including a simple log, map and message creator

SIDC			=	Symbol identification code (APP-6A/B/MIL-STD-2525B/C/STANAG 2019)

ASCII			=	American Standard Code for Information Interchange – in this context the ISO-8859-1 table is meant

BASE64		=	Binary-to-text encoding scheme, which is using an alphabet of 64 characters

- General connection guidelines

- Common conventions

- Basic format is CSV using ‘;’ (0x3B) as separation character between two data fields, excess semicolons can be truncated

- Elements of list values are separated by ‘#’ (0x23), List values are marked with a ‘*’ in the specification

- Message ends with a ‘\n’ (0x0A) as termination

- Mandatory fields/elements except the name are marked with (M)

- Maximum length of untyped or text fields is 256 bytes

- The messages are human-readable and using the ASCII-table

- Fields with (Binary)Data which possibly contains a special character (e.g. 0x0A, 0x23, 0x3B) has to be encoded with BASE64

- Unknown/Invalid values must not be transmitted, the respective field will be left empty

- All timestamps are generally 64-bit integer values, corresponding to UNIX standard timestamp in milliseconds

- Support for IPv4 and IPv6 (except for serial connection)

- SEC/MockUp/Applications can send and receive at any time

- Application shall send heartbeat message not more often than with 1Hz (+-100ms), but can vary if it is required

- SEC/MockUp answers heartbeat also with a heartbeat message (see chapter IV.2.12)

- Authentication

- If authentication/encryption is required, it’s always preferred to use VPN connections instead

- Messages could be authenticated by fulfilling the MAC field using a shared password (see chapter IV.1.1)

- The password can either be defined in advance or exchanged using the Diffie-Hellman process and the KEYEXCHANGE message (see chapter IV.2.14). It’s recommended to authenticate even this message with a pre-shared password.

- For calculating the MAC you have to use the complete message incl. header and setting the MAC temporary to “0000”

- To save data, it is possible to limit the MAC to the first 4 bytes/32 bits while reducing security

- At minimum standards defined by FIPS 140-2 (Federal Information Processing Standard) have to be used
That means it is preferred to use:

  - 32Bit/256Bit HMAC in combination SHA256 (FIPS 198-1 / RFC 2104)

  - 32Bit/128Bit CMAC in combination AES128 (NIST SP 800-38B)

  - 32Bit/128Bit GMAC in combination AES128 (NIST SP 800-38D)

- It’s recommended to use MAC DBRG (Deterministic Random Bit Generator, NIST SP 800-90A/B)

Sample (32Bit CMAC, Password:expressexpressex): OWNUNIT;5E;0195236D151A;66A3;R;;089A01E7;53.32;8.11;0;5.5;21;22;;;FGS Bayern

- Encryption

- Encryption is optional and as already wrote, if authentication/encryption is required, it’s preferred to use VPN instead

- At minimum standards defined by FIPS 140-2 (Federal Information Processing Standard) have to be used
That means it is preferred to use:

  - AES128/256 CFB/NoPadding (NIST SP 800-38A)

  - AES128/256 CTR/NoPadding (NIST SP 800-38A)

  - AES128/256 ECB/PKCS7Padding (NIST SP 800-38A)

  - XOR (Pseudo-encryption ONLY for testing/debugging purposes or for very light obfuscation)

- It’s recommended to use MAC DBRG (Deterministic Random Bit Generator, NIST SP 800-90A/B)

- Encrypted data must be BASE64 encoded - all incl. header (see chapter IV.1.1.1) have to be encoded

- If there is password given every message have to be encrypted – mixture of encrypted and plain message is not allowed

Sample reference message:
OWNUNIT;5E;01952384BD8D;66A3;R;;;53.32;8.11;0;5.5;21;22;;;FGS Bayern;sfspclff-------

Sample encrypted message (AES128, CFB, Password:expressexpressex):
UdlsDIB4oeKiuU4PXtV9qCPrrk10yPFg38Bakm7oGVOOC3siuczsyg37+Q9eiDE1Z0qyaXQl4puRGdB0mpb1Vf6cfhj7n7270Gv/VjW5Ol8IAFJh

- Compression

- Compression is optional (has to be used BEFORE an optional encryption because of the high entropy!)

- Use the widely spreaded “deflate” for compression (only effective if the message size exceeds 140 characters due to the BASE64 encoding)

- Compressed data must be BASE64 encoded – all incl. header (see chapter IV.1.1.1) have to be compressed

- If the first bytes of a received message don’t match a message name, prove for compression

Sample reference message:

TEXT;D3;01952381E21B;324E;S;TRUE;;;;1;NONE;"This is an alert!"

Sample compressed message:

C3GNCLF2MbY2MLQ0NTK2MHQ1MnSyNjYycbUOtg4JCnW1BgJDaz9/P1drpZCMzGIFIErMU0jMSS0qUVQCAA==

- Possible connection methods

In this chapter you will find a kinds of connections which are available to exchange data with the UNIITY SEDAP-Express connector. The most simple way is to use TCP, which is reliable and has not the limits of the free text or data field in some messages.

  - TCP Connection

- Standard port 50000, but customizable

- SEC/SECMockUp can play server or client role

- One should use only one TCP connection for any kind of message and keep it open

  - UDP-Connection

- Standard port 50000, but customizable

- Support for Uni-, Broad- or Multicast mode, multiple messages per UDP-packet possible

- Standard Multicast-Address is 228.2.19.80 resp. ff02:8:2:19:80::1

- HEARTBEAT messages should be answered with UDP-Unicast if possible (see chapter IV.2.12)

- Is highly recommended to use the <number> and <acknowledgement> field in the header cause of possible packet lost

  - Serial connection

- Standard 115200-8-N-1, Full-Duplex is preferred, Half-Duplex and Simplex without acknowledge requests

- There are three modes available, depending on the use case:

  - Message never contains a \n (0x0A), therefore the message could be sent with an additional appended \n\n\n\n

  - Message contains one or more \n (0x0A), have to be BASE64-encoded and sent with an additional appended \n\n\n\n

  - MQTT connection

- Connection can establish via normal TCP/MQTT or via SSL/Certificates protocol

- Messages have to be published under SEDAP-X/<senderid>/<messagetype>

  - REST-API

- Standard ports are HTTP 80 or HTTPS  443, but customizable

- Deflate or GZip compression should be supported

  - Protocol Buffers

- Ports and other TCP or UDP parameters are the same as descripted in chapter III.5 and III.6.

- Specifications of the data exchange

- General specifications

To make things easy, SEDAP-Express offers various kinds of connections. You are free to choose which variant best suits your needs, with which you have the most experience, or for which customizable code already exists.

  - TCP-/UDP-/Serial connections

On principle, messages have a CSV structure with a common header:

<Name>(M);<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<Content>

Everything except the name is basically optional. Certain messages require specific header elements, such as time or the sender.
These exceptions are described in the chapter of the respective message. Hexadecimal numbers have no “0x”-prefix.

| <Name> | Defines the purpose of a message. Sometimes it’s so-called topic. |
| --- | --- |
| <Number> | This is a hexadecimal string representation of a 7-bit sequential number. Each type of message has its own counter that starts again with zero after reaching 127/7F. A reconnect does not resets the counters! The limitation to 7-bit or a max of 127 prevents problems with signed/unsigned byte formats. |
| <Time> | A hexadecimal string representation of a 64-bit Unix time stamp with milliseconds. |
| <Sender> | You can use freely selected textual identifiers, e.g. “OKRA”. This field won’t be changed, even if a message has been forwarded or relayed. This sender identification can be chosen by the participants themselves or permanently assigned by a responsible institution when preparing a specific use/network. If information of a sub-system has to be forwarded the sender identification should be the source of the original information, in that case of the sub-system. For instance, if central unit relays information of drones in a swarm. |
| <Classification> | Classification level of the content: P=public, U=unclassified, R=restricted, C=confidential, S=secret, T=top secret |
| <Acknowledgement> | TRUE=request an acknowledgement, FALSE/Nothing=No acknowledgement. An acknowledgement of a messages requires a specified message number (second field of this header)! |
| <MAC> | A 32/128/256-bit wide hash-based message authentication code for verification (see chapter III.2 for more) |
| <Content> | Content of the message, depending on the message purpose. |

  - REST-API connection

If the REST-API shall be used, it’s preferred to also use the provided JSON schema file and the generated code which either comes with the SEDAP-Express SDK or has been generated by yourself. You can find the schema and sample requests/answers in chapter IV.3 or on https://SEDAP.Express.

  - Protobuf connection

At least it’s also possible to use Google™ protocol buffers to exchange the SEDAP-Express messages. You can find the schema in chapter IV.4 or on https://SEDAP.Express. This allows you to generate your own code or use the existing code from the framework or the sample client.

- Message specifications

This is the list of all so-far available messages and their structures and content including some samples. All units of measurement are generally given in SI-units, but there are deviations where this makes sense due to the usual range of values. In the following the used units will be given within square brackets for all message-descriptions. The altitude is the altitude above sea-level. A value of zero means exactly on ground, if the position is on land. Latitude and longitude are in decimal degrees, while positive values means north and east respective negative values south and west. Relative position values are defined this way, that the x-axis points to the east direction, y-axis points to the north and the z-axis is equal to the height above the unit. Speed and course are meant to be relative to ground. Course and heading have a range from zero to 359.9999, are relative to geographic north (zero degree) and rotate clockwise. In general, all values are optional. mandatory parameters are marked with (M). In general, numerical values are rational numbers (floating point), unless otherwise defined. In the examples, fields are sometimes intentionally shortened because they would otherwise be too large. This is the case, for example, with the KEYECHANGE message.

  - OWNUNIT

Description: Position, movement, and identification data of your own unit. This can be a base station, a C2 center, a drone, a vehicle,
a person, or the host device on which the client is installed.

When a client sends this message, it is converted into a contact and sent to the UNIITY network. If there is more than one “own unit,” the “Sender” field must be filled in to distinguish between the different units. In exceptional cases, different names can also be used for differentiation. The corresponding option must then be explicitly specified in the SEC.

Structure:
OWNUNIT;<Number>;<Time>;<Sender>(M?);<Classification>;<Acknowledgement>;<MAC>;
<Latitude>[°](M);<Longitude>[°](M);<Altitude>[m];
<Speed over ground>[m/s];<Course over ground>[°];
<Heading>[°];<Roll>[°];<Pitch>[°];
<Name>;<SIDC>

Sample 1: OWNUNIT;5E;0191C643A8AF;DRONEONE;R;;;53.32;8.11;0;5.5;21;22;;;FGS Bayern;SFSPCLFF-------

Sample 2: OWNUNIT;11;0191C643A8AF;22AA;U;FALSE;4389F10D;77.88;-10.12;5577.0;33.44;55.66;1.1;-2.2;3.3;Ownunit;SFGPIB----H----

  - CONTACT

Description: Positional, kinematic and identification data of a contact. For example, this message would be used by a sensor to report a contact it recognized. In return this message would be used to receive the tactical picture from the UNIITY network. You have to choose between Lat/Lon/Alt OR relative distance – one is mandatory. If you choose relative distance, you must also provide an OWNUNIT (Chapter 2.1) message – otherwise the position of the receiver will be used as reference point. Make sure to implement Delete as well!

Structure:
CONTACT;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;
<ContactID>(M);<DeleteFlag>;<Latitude>[°](M);<Longitude>[°](M);<Altitude>[m];
<relative X-Distance>[m](M);< relative Y-Distance>[m](M);<relative Z-Distance>[m](M);
<Speed over ground>[m/s];<Course over ground>[°];
<Heading>[°];<Roll>[°];<Pitch>[°];
<width>[m];<length>[m];<height>[m];
<Name>;<Source>;<SIDC>;<MMSI>;<ICAO>;<MediaData>;<Comment>

| ContactID | ASCII | A positive identification unique number or free text id of the contact, chosen by the sender of this message. This number should be unique at least throughout the entire period of, for example, the exercise. |
| --- | --- | --- |
| DeleteFlag | TRUE | Contact has to be removed |
|  | FALSE | Contact is current |
| Name | ASCII | Name of the contact, maximum length 64 bytes |
| Source | ASCII | Available types (more than one available): R=Radar, A=AIS, I=IFF/ADS-B, S=Sonar, E=EW, O=Optical, Y=Synthetic, M=Manual |
| SIDC | SIDC | Identification code, could be written in lower or upper case (starting with “s”) |
| MMSI | MMSI | Maritime Mobile Service Identity |
| ICAO | ICAO | International Civil Aviation Organization |
| MediaData | BASE64 | Image/Video/Sound URL or data (JPG, PNG, TIF, MP4, TS, WAV, etc. ) encoded in BASE64, preferred length up to 65000 bytes when using UDP |
| Comment | BASE64 | Free text to the contact, maximum length 8192 bytes |

Sample 1: CONTACT;5E;0191C643A8AF;83C5;R;;;100;FALSE;53.32;8.11;0;;;;120;275;;;;;;;FGS Bayern;AR;SFSPCLFF-------;;;;VXNlIENIMjI=

Sample 2: CONTACT;5F;0191C643A8AF;83C5;U;;;101;;36.32;12.11;2000;;;;44;;;;;;;;Unknown;O;;221333201;;;UG9zcyBOZXRoZXJsYW5kcw==
Sample 3: CONTACT;;0191C643A8AF;4371;S;;;102;;53.32;8.11;;;;;;;;;;;;;PossTank;;;;;XFxzcnZcc25kXDU0aDJoLndhdg==;cGxzIGhlYXI=

  - POINT

Description: Positional, kinematic and identification data of a (geographical) point (e.g. a person over board, bridge, hill, meeting point something like that). You have to choose between Lat/Lon/Alt OR relative distance – one is mandatory. If you choose relative distance, you must also provide an OWNUNIT (Chapter 2.1) message – otherwise the position of the receiver will be used as reference point. Make sure to implement Delete as well!

Structure:
POINT;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;
<ContactID>(M);<DeleteFlag>;<Latitude>[°](M);<Longitude>[°](M);<Altitude>[m];
<relative X-Distance>[m](M);< relative Y-Distance>[m](M);<relative Z-Distance>[m](M);
<Speed over ground>[m/s];<Course over ground>[°];
<Heading>[°];<Roll>[°];<Pitch>[°];
<width>[m];<length>[m];<height>[m];
<Name>;<SIDC>;<MediaData>;<Comment>

| PointID | ASCII | A positive identification unique number or free text id of the contact, chosen by the sender of this message. |
| --- | --- | --- |
| DeleteFlag | TRUE | Point has to be removed |
|  | FALSE | Point is current |
| Name | ASCII | Name of the point, maximum length 64 bytes |
| SIDC | SIDC | Identification code, could be written in lower or upper case (starting with “g”) |
| MediaData | BASE64 | Image/Video/Sound URL or data (JPG, PNG, TIF, MP4, TS, WAV, etc. ) encoded in BASE64, preferred length up to 65000 bytes when using UDP |
| Graphics are currentComment | BASE64 | Free text to the contact, maximum length 8192 bytes |

Sample 1: POINT;6A;0013E45956BE;59CE;U;;FFAA327B;1000;FALSE;;;;100;130;0;2;30;;;;;;;Person in water;gfopep---------

Sample 2: POINT;5E;0000661D4410;66A3;R;;;100;FALSE;53.32;8.11;0;;;;120;275;;;;;;;Target Alpha;gffppt---------;;VGFyZ2V0IHBvaW50

  - EMISSION

Description: With this message one can report the positional data, emission attributes and additional identification information of an electro-magnetic, optical or acoustic emission or the bearing or visually recognizable objects.

Structure:
EMISSION;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;
<EmissionID>(M);<DeleteFlag>;

<SensorLatitude>[°](M);<SensorLongitude>[°](M);<SensorAltitude>[m];
<EmitterLatitude>[°];<EmitterLongitude>[°];<EmitterAltitude>[m];
<Bearing>[°](M);<Frequencies[Hz]*>;<Bandwidth[Hz]>;<Power[db(A)]>;<FreqAgility>;<PRFAgility>;
<Function>;<SpotNumber>;<SIDC>;<Comment>

| EmissionID | ASCII | A positive identification unique number or free text id of the emission, chosen by the sender of this message. This number should also be unique with regard to contact numbers and similar ID numbers. |
| --- | --- | --- |
| DeleteFlag | TRUE | Emission has to be removed |
|  | FALSE | Emission is current |
| FreqAgility | 00 | Stable Fixed |
|  | 01 | Agile |
|  | 02 | Periodic |
|  | 03 | Hopper |
|  | 04 | Batch Hopper |
|  | 05 | Unknown |
| PRFAgility | 00 | Fixed periodic |
|  | 01 | Staggered |
|  | 02 | Jittered |
|  | 03 | Wobbulated |
|  | 04 | Sliding |
|  | 05 | Dwell switch |
|  | 06 | Unknown PRF |
|  | 07 | CW |
| Function | 00 | Unknown |
|  | 01 | ESM Beacon/Transponder |
|  | 02 | ESM Navigation |
|  | 03 | ESM Voice Communication |
|  | 04 | ESM Data Communication |
|  | 05 | ESM Radar |
|  | 06 | ESM IFF/ADS-B |
|  | 07 | ESM Guidance |
|  | 08 | ESM Weapon |
|  | 09 | ESM Jammer |
|  | 0A | ESM Natural |
|  | 0B | ACOUSTIC Object |
|  | 0C | ACOUSTIC Submarine |
|  | 0D | ACOUSTIC Variable Depth Sonar |
|  | 0E | ACOUSTIC Array Sonar |
|  | 0F | ACOUSTIC Active Sonar |
|  | 10 | ACOUSTIC Torpedo Sonar |
|  | 11 | ACOUSTIC Sono Buoy |
|  | 12 | ACOUSTIC Decoy Signal |
|  | 13 | ACOUSTIC Hit Noise |
|  | 14 | ACOUSTIC Propeller Noise |
|  | 15 | ACOUSTIC Underwater Telephone |
|  | 16 | ACOUSTIC Communication |
|  | 17 | ACOUSTIC Noise |
|  | 18 | LASER Range Finder |
|  | 19 | LASER Designator |
|  | 1A | LASER Beam Rider |
|  | 1B | LASER Dazzler |
|  | 1C | LASER Lidar |
|  | 1D | LASER Weapon |
|  | 1E | VISUAL Object |
| SIDC | SIDC | Identification code |
| Comment | BASE64 | Free text to the emission, maximum length 65000 bytes |

Sample 1: EMISSION;5E;0195238E15AD;66A3;R;;;100;;53.32;8.11;0;;;;20;8725000.0#8735000.0;20000;3;0;2;6;10233;;SA-8

Sample 2: EMISSION;5F;0195238E25AD;66A3;R;;;101;;54.86;9.32;0;52.12;9.8;50;233;25725.0;4000;1;5;2;0;;sngpesr--------

  - METEO

Description: Metrological data of the environment. If you also send the OWNUNIT, then UNIITY will link this metrological data with the last position from the OWNUNIT. The alternative is to give a reference to a contact, point or graphic.

Structure:
METEO;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;
<SpeedThroughWater>[m/s];<WaterSpeed>[m/s];<WaterDirection>[°];<WaterTemperature>[°C];<WaterDepth>[m];
<AirTemperature>[°C];<DewPoint>[°C];<HumidityRel>[%];<Pressure>[hPa];<WindSpeed>[m/s];<WindDirection>[°];
<Visibility>[km];<CloudHeight>[m];<CloudCover>[%];Reference

Sample 1: METEO;1C;0195238E25AD;74BE;U;;;15.4;15.5;;;;;10.2;72;1005;25;111;50;2500;33;RefPoint1

Sample 2: METEO;;0195238E25AD;;U;;;23.2;100;;;;10.2;72;80;998;20;;;500;100;1000

  - TEXT

Description: Human readable textual data. This could be an alert message, but also a simple text message for chatting. If the text possibly contains special characters (e.g. UTF-x, 0x0A, 0x23, … see chapter III.1), it has to be BASE64-encoded and the encoding indicator has to be set. If the coding indicator is not set, no coding is assumed. If the message for example is linked to a contact, the corresponding CONTACT message should be sent first. If the recipient field is left empty, this means everyone (as with a broadcast).

Structure:

TEXT;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<Recipient>;<Type>;<Encoding>;<Text>(M);Reference

| Recipient | ASCII | You can use a freely selected textual identifiers, as explained in the table from chapter IV.1.1. |
| --- | --- | --- |
| Type | 00 | Undefined |
|  | 01 | Alert |
|  | 02 | Warning |
|  | 03 | Notice |
|  | 04 | Chat |
| Encoding | BASE64 | Text is BASE64 encoded |
|  | NONE | Text is not encoded |
| Text | ASCII | Free text of the message, maximum length 65000 bytes |
| Reference | ASCII | Reference to a contact, point or emission |

Sample 1: TEXT;78;0195238E25AD;324E;S;TRUE;;;1;NONE;"This is an alert!";1000

Sample 2: TEXT;79;0195238E25CC;324E;C;TRUE;;;2;NONE;"This is a warning!"

Sample 3: TEXT;7A;0195238E25EF;324E;R;;;;3;;"This is a notice!"

Sample 4: TEXT;7B;0195238E285B;324E;U;;;ORKA;4;BASE64;IlRoaXMgaXMgYSBjaGF0IG1lc3NhZ2UhIg==

  - GRAPHIC

Description: Define graphical plans likes polygons, squares or routes. You can also use it to transmit the current field of view of the camera, in what directions the sensor or weapon is heading or what is the range area of the weapon system. Please note, that in lists (# symbol, e.g. Path) the elements like latitude and longitude within an list element are separated by “,” and not by “;”.

Structure:
GRAPHIC;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<GraphicID>(M);<DeleteFlag>;
<GraphicType>(M);<LineWidth>;<LineColor>;<FillColor>;<TextColor>;<Encoding>;<Annotation>;<additional GraphicType-dependent parameters>*

| GraphicID | ASCII | A positive identification unique number or free text id of the graphic, chosen by the sender of this message. This number should also be unique so that updates of an existing graphic is possible. |
| --- | --- | --- |
| DeleteFlag | TRUE | Graphic has to be removed |
|  | FALSE | Graphic is current |
| GraphicType | 00 | Point: <Latitude>[°];<Longitude>[°];<Altitude>[m] |
|  | 01 | Path: <Latitude>[°],<Longitude>[°],<Altitude>[m]# … |
|  | 02 | Polygon: <Latitude>[°],<Longitude>[°],<Altitude>[m]# … |
|  | 03 | Rectangle: <Latitude>[°],<Longitude>[°],<Altitude>[m];<Width>[m];<Length>[m];<Rotation>[°] |
|  | 04 | Square: <Latitude>[°];<Longitude>[°];<Altitude>[m];<Width>[m];<Rotation>[°] |
|  | 05 | Circle: <CenterLatitude>[°];<CenterLongitude>[°];<CenterAltitude>[m]; <Radius>[m];<StartAngle>[°];<EndAngle>[°] |
|  | 06 | Ellipse: <CenterLatitude>[°];<CenterLongitude>[°];<CenterAltitude>[m]; <RadiusX>[m];<RadiusY>[m];<Rotation>[°] |
|  | 07 | Block: <Latitude>[°];<Longitude>[°];<Altitude>[m];<Width[m];<Lenght>[m];<Height>[m]; <RotationX>[°];<RotationY>[°];<RotationZ>[°] |
|  | 08 | Sphere: <Latitude>[°];<Longitude>[°];<Altitude>[m];<Radius>[m] |
|  | 09 | Ellipsoid: <CenterLatitude>[°];<CenterLongitude>[°];<CenterAltitude>[m]; <RadiusX>[m];<RadiusY>[m];<RadiusZ>[m];<RotationX>[°];<RotationY>[°];<RotationZ>[°] |
|  | 0A | SensorFieldOfView: <Azimuth>[°];<Elevation>[°];<Latitude>[°],<Longitude>[°],<Altitude>[m]# … |
|  | 0B | WeaponFieldOfFire: <Azimuth>[°];<Elevation>[°];<Latitude>[°],<Longitude>[°],<Altitude>[m]# … |
| LineWidth | => 1 | Width of the line or the point |
| LineColor | RGBA | Color of the line in Web notation 800000FF for a darker red |
| FillColor | RGBA | Color of the filling in Web notation 00FF0080 for translucent green |
| TextColor | RGBA | Color of the text in Web notation 32CD32FF for lime |
| Encoding | BASE64 | Text is BASE64 encoded |
|  | NONE | Text is not encoded |
| Annotation | ASCII | Text for an annotation to this graphic, maximum length 32 bytes |

Sample 1: GRAPHIC;79;0195238E35AD;910E;U;;;FFDA;;08;1;FF800000;;;BASE64;QXJlYSBBbHBoYQ==;53.43;9.45;0;1000

Sample 2: GRAPHIC;78;0195238E45AD;910E;U;;;A327;;01;1;80808000;;FFFF0000;;Transit;54.23,12.86#54.30,12.9#54.55,13.30

  - COMMAND

Description: Command for one specific or all possible recipients. Which camera is assigned to which number and which camera modes are available must be defined specifically for each application and depending on the camera/sensor platform. The same also applies in particular for the generic action and any additional parameters for it. Time stamps are optional and in milliseconds. If no time stamp is given, it’s interpreted as instantly. Of course, you cannot use the generic SEDAP-Express connector in these cases, but must implement a specific and user-defined connector. Unix times stamps have to be written as hexadecimal string. If the vehicle shall follow or engage a e.g. contact, the position of this contact has to be transmitted frequently using the CONTACT message. Depth is equivalent to altitude when commanding submersible vehicles and always a positive value. The camera modes have the following meanings: DL = Daylight, IR = Infrared, and LI = Light Intensifier.

Structure:

COMMAND;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;
<Recipient>(M);<CmdID>;<CmdFlag>(M);<CmdExTime>;<CmdType>(M);<additional cmdType-dependent parameters>*

| Recipient | ASCII | You can use a freely selected textual identifiers, as explained in the table from chapter IV.1.1. |
| --- | --- | --- |
| CmdID | HexString | 16-Bit hexadecimal identifier of the command (0000 means all last commands) |
| CmdFlag | 00 | Add command |
|  | 01 | Replace (last) command |
|  | 02 | Cancel (last) command |
|  | 03 | Cancel all commands (Same as 02 if there it is not a command sequence) |
| CmdExTime | Long | Unix time stamp when the command shall be executed |
| CmdType | 00 | Power off: <Power on unix time stamp>(optional) |
|  | 01 | Restart |
|  | 02 | Standby: <Wakeup unix time stamp>(optional) |
|  | 03 | Sync time: <IP/Hostname of a NTP server> |
|  | 04 | Calibrate gyro |
|  | 05 | Calibrate compass |
|  | 06 | Send status |
|  | 07 | Set manual mode |
|  | 08 | Set semi-autonomous mode |
|  | 09 | Set autonomous mode |
|  | 0A | Set failsafe mode |
|  | 10 | Start engine |
|  | 11 | Test engine |
|  | 12 | Set engine power: <PowerLevel>[0-100%] |
|  | 13 | Stop engine |
|  | 14 | Stop movement |
|  | 15 | Toggle lights: <Status>[ON ¦ OFF];<BrightnessLevel>[0-100%];<StrobePeriod>[s] |
|  | 16 | Deploy parachute |
|  | 20 | Set heading: <HeadingAngle>[°] |
|  | 21 | Set altitude: <Altitude>[m] |
|  | 22 | Set speed: <Speed>[m/s] |
|  | 23 | Rotate: <HeadingAngle>[°];<RollAngle>[°];<PitchAngle>[°] |
|  | 24 | Move to: <Latitude>[°];<Longitude>[°];<Altitude>[m];<Tolerance>[m];<Unix time stamp> |
|  | 25 | Follow contact: <ContactID> |
|  | 26 | Return home: <Tolerance>[m];<Unix time stamp> |
|  | 27 | Set home location: <Latitude>[°];<Longitude>[°];<Altitude>[m];<Tolerance>[m] |
|  | 28 | Take off: <Latitude>[°];<Longitude>[°];<Direction>[°] |
|  | 29 | Land: <Latitude>[°];<Longitude>[°];<Direction>[°] |
|  | 2A | Submerge: <Depth>[m] |
|  | 2B | Surface |
|  | 2C | Dock: <Latitude>[°];<Longitude>[°];<Direction>[°] |
|  | 30 | Loiter/Orbiting: <CenterLatitude>[°];<CenterLongitude>[°];<Altitude>[m];<Radius>[m] |
|  | 31 | Scan: <Latitude>[°];<Longitude>[°];<Altitude>[m] |
|  | 32 | Scan area: <Latitude1>[°];<Longitude1>[°];<Latitude2>[°];<Longitude2>[°];<Altitude>[m];<RotationAngle>[°] |
|  | 33 | Take photo: <CameraID> |
|  | 34 | Record video: <CameraID>;<ON¦OFF>;<Duration> |
|  | 35 | Stream video: <CameraID>;<ON¦OFF> |
|  | 36 | Set camera parameters: <CameraID>;<Zoom>[0-100%];<Mode>[DL ¦ IR ¦ LI] |
|  | 37 | Set orientation of camera: <CameraID>;<Azimuth>[°];<Elevation>[°] |
|  | 40 | Actuator check: <ActuatorID> |
|  | 41 | Set orientation of actuator: <ActuatorID>;<Azimuth>[°];<Elevation>[°] |
|  | 42 | Actuator pick up object: <ActuatorID> |
|  | 43 | Actuator release object: <ActuatorID> |
|  | 50 | Pre-arm check: <WeaponID> |
|  | 51 | Arm: <WeaponID> |
|  | 52 | Disarm: <WeaponID> |
|  | 53 | Set orientation of weapon: <WeaponID>;<Azimuth>[°];<Elevation>[°] |
|  | 54 | StartEngagement: <WeaponID>;<ContactID ¦ PointID ¦ EmmisionID ¦ GraphicID>(M) |
|  | 55 | HoldEngagement: <WeaponID>;<ContactID ¦ PointID ¦ EmmisionID ¦ GraphicID>(M) |
|  | 56 | StopEngagement: <WeaponID>;<ContactID ¦ PointID ¦ EmmisionID ¦ GraphicID>(M) |
|  | EE | Sanitize system (e.g. in a case of emergency or drone hijacking) |
|  | EF | Self destruction |
|  | FF | Generic Action: <Kind of action>[String] (Has to be defined individually, inclusive implementing a custom connector software for UNIITY) |

Sample 1: COMMAND;55;0195238E25AD;5BCD;S;TRUE;4389F10D;ORKA;1111;01;54;bomb;1000
Sample 2: COMMAND;29;0195238E35AD;E4B3;C;TRUE;;Drone1;;FF;00;OPEN_BAY

Sample 3: COMMAND;29;0195238F55AD;E4B3;C;TRUE;;Drone2;0000;03    (Cancel all last commands)

Sample 4: A complex move to with loitering at the end. The drone must calculate the speed and course itself based on the timestamp of the last move.

COMMAND;29;0195238E25AD;E4B3;C;TRUE;;ORKA;0331;00;;24;53.5143;8.1574;50;5

COMMAND;2A;0195238E25AD;E4B3;C;TRUE;;ORKA;0331;00;;24;53.4897;8.1908;50;5

COMMAND;2B;0195238E25AD;E4B3;C;TRUE;;ORKA;0331;00;;24;53.4694;8.2131;50;5

COMMAND;2C;0195238E25AD;E4B3;C;TRUE;;ORKA;0331;00;;24;53.4397;8.2262;50;5;019D25300FC0 (approx. 15 minutes later)

COMMAND;2C;0195238E25AD;E4B3;C;TRUE;;ORKA;0331;01;019D75300FFF;34;CAM1;ON;3600

COMMAND;2E;0195238E25AD;E4B3;C;TRUE;;ORKA;0331;00;;30;53.4397;8.2262;1000;200

  - STATUS

Description: This message transports some kinds of status variables like the remaining batterie capacity and optionally the execution status of the last or a specific command (see chapter IV.2.7). If the status information of a sub-system has to be forwarded, the sender identification should be the source of the original information, meaning for swarm of drones, the concrete, individual drone identification. The storage level can mean the memory for recordings, but also the physical storage room when picking up objects or transfer things.

Structure:
STATUS;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<TecStatus>;<OpsStatus>;<AmmunitionLevels>*;
<FuelLevels>*;<BatteryLevels>*;<StorageLevels>*;<CmdID>;<CmdState>;<IP/Host>;<Media>*;<Text>

| TechnicalStatus | 0 | Off/Absent |
| --- | --- | --- |
|  | 1 | Initializing |
|  | 2 | Degraded |
|  | 3 | Operational |
|  | 4 | Fault |
| OpsStatus | 0 | Not operational |
|  | 1 | Degraded |
|  | 2 | Operational |
|  | 3 | Operational (semi-autonomous) |
|  | 4 | Operational (autonomous) |
| AmmunitionLevels | String#% | Relative remaining ammunition: <name of the weapon>#<ammunition level>#.... |
| FuelLevels | String#% | Relative remaining fuel capacity: <name of the fuel tank>#<fuel level>#.... |
| BatteryLevels | String#% | Relative remaining battery capacity: <name of the battery>#<battery level>#.... |
| StorageLevels | String#% | Relative remaining storage capacity:<name of storage>#<storageLevel> |
| CmdID | HexString | Identifier of the command in the related COMMAND message (IV.2.8) |
| CmdState | 00 | Undefined |
|  | 01 | Executed successfully |
|  | 02 | Partially successfully executed |
|  | 03 | Not successfully executed |
|  | 04 | Execution not possible (yet) |
|  | 05 | Will be executed at ;<timestamp> |
| IP/Host | BASE64 | IP or hostname of the platform, maximum length 64 bytes |
| Media | BASE64 | List of video stream or image URLs, maximum length 4096 bytes |
| Text | BASE64 | Human readable free text description of the status |

Sample 1: STATUS;15;0195238E25AD;75DA;U;;;4;2;MLG#20;;Accu1#50;;443D;1;MTAuMC4wLjEzMg==;;RnVsbHkgb3BlcmF0aW9uYWw=

Sample 2: STATUS;16;0195238E25AD;129E;R;;;2;2;BMG#10;;;;ED32;3;;aHR0cDovLzEwLjAuMC4xL2ltYWdlLnBuZw==;T3V0IG9mIGZ1ZWwh

  - ACKNOWLEDGE

Description: If a client or the SEC requested an acknowledge of a packet one has to use this this message. The acknowledgement flag is fixed set to FALSE. The awaiting client or SEC have to wait maximal one seconds before resending the original message with set acknowledgement flag.

Structure:

ACKNOWLEDGE;<Number>;<Time>;<Sender>;<Classification>;;<MAC>;
<Recipient>(M);<Type of the message>(M);<Number of the message>(M)

| Recipient | ASCII | You can use a freely selected textual identifiers, as explained in the table from chapter IV.1.1. |
| --- | --- | --- |
| Type | ASCII | The type of the message which should be acknowledged (e.g. CONTACT, RESEND, …). |
| Number | HexString | The number of the message which should be acknowledged. This is a hexadecimal string representation of an 7-bit. |

Sample: ACKNOWLEDGE;18;0195238E25AD;129E;R;;;LASSY;COMMAND;2B

  - RESEND

Description: Missing messages can be requested again with this message. These messages can be recognized by the message number in the header, the sender ID and the message name as described in chapter IV.1.1.1.

Structure:

RESEND;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;
<Recipient>(M);<Name of the missing message>(M);<Number of the missing message>(M)

| Recipient | ASCII | You can use a freely selected textual identifiers, as explained in the table from chapter IV.1.1. |
| --- | --- | --- |
| Name | ASCII | The name of the message which should resend. |
| Number | Number | The number of the message which should resend. This is a hexadecimal string representation of a 7-bit integer. |

Sample: RESEND;20;0195238E25AD;129E;R;;;FE2A;TEXT;5D

  - GENERIC

Description: This message is an empty container for transporting any kind of data. It has to be defined in the respective case. For example, one can use it to exchange the original UNIITY/SEDAP messages or other propriety protocol data. In the last case you have to use any other self-defined type. If the coding indicator is not set, no coding is assumed.

Structure:

GENERIC;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;

<ContentType>;<Encoding>;<Content>

| ContentType | SEDAP | Content is an original SEDAP message |
| --- | --- | --- |
|  | ASCII | Content is a custom-defined ASCII string |
|  | NMEA | Content is a NMEA0183 string |
|  | XML | Content is in an XML structure |
|  | JSON | Content is JSON formatted |
|  | BINARY | Self-defined binary array |
| Encoding | BASE64 | Content is BASE64 encoded |
|  | NONE | Content is NOT encoded |
| Content |  | Any content in printable ASCII or BASE64 encoded data, maximum length 8192 bytes |

Sample 1: GENERIC;5E;0195238E25AD;66A3;R;;;JSON;;{"object": {"x": "1","y": "2"},"string": "Hello World"}

Sample 2: GENERIC;5E;0195238E25AD;66A3;R;TRUE;;BINARY;BASE64;U2FtcGxlIGJpbmFyeSBkYXRhIEdyZWV0aW5ncyA6RA==

Sample 3: GENERIC;5E;0195238E25AD;66A3;R;;;NMEA;NONE;$RATTM,11,11.4,13.6,T,7.0,20.0,T,0.0,0.0,N,,Q,,154125.82,A,*17

  - HEARTBEAT

Description: This message offers the possibility to check the connection, which is primarily important, if you are using UDP or serial connection. It should not be sent more often than 1Hz. Nevertheless, if it is needed – one can use a faster repetition. The recipient field is optional and can be one single recipient or a list of more than one recipient. If no recipient is provided than all possible recipients in the network/serial net are addressed. A heartbeat message has an empty acknowledgement flag, because you cannot request one for it. Besides this, the acknowledgement flag is fixed set to FALSE (empty field).

Structure:
HEARTBEAT;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<Recipient>

| Recipient | ASCII | You can use a freely selected textual identifiers, as explained in the table from chapter IV.1.1. |
| --- | --- | --- |

Sample 1: HEARTBEAT;42;0195238E25AD;89AD;U;;;ORKA

Sample 2: HEARTBEAT;43;;1022

Sample 3: HEARTBEAT;43;

Sample 4: HEARTBEAT

  - TIMESYNC

Description: A correctly synchronized time often plays an important role. In general, you should use operating system functions/command line tools and a dedicated time server. But if this is not available for reasons, there is this message dedicated for this purpose. If necessary, a client should perform time synchronization himself from time to time.

If a TIMESYNC message is received, the current system time of the recipient must be included as timestamp in the response. The original sender can then calculate the round-trip time (RTT) from half the difference between the time the response was received and the time the message was sent and add it to the transmitted timestamp. The result can then be adopted as the new system time.

Structure:
TIMESYNC;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<Timestamp>

| Timestamp | HexString | A hexadecimal string representation of a 64-bit Unix time stamp with milliseconds. |
| --- | --- | --- |

Sample 1: TIMESYNC;42;0191C643A8AF;89AD;U

Sample 2: TIMESYNC;12;;FE2A;U;;;0191C643A8AF

  - KEYEXCHANGE

Description: If you don’t have the possibility to exchange a password/key via another channel (e.g., mail, telco), this message can be used to exchange keys via Diffie-Hellman-Merkle or via the post-quantum Kyber-/FrodoKEM process. It’s preferred to use ECDH or standard DH with MAC DRBG. If possible, also use MAC authentication for these messages or otherwise do some plausibility checks. The current phase defines if a field is mandatory. Both sides can restart the process by simply sending a message with the phase set again to zero again. The other side should restart the whole process inclusive generating new key pairs if using ECDH or Kyber-/FrodoKEM. Please pay attention, that the ECDH has no phase 1. The whole process always ends by sending a finalization confirmation.

To generate a 128- or 256-bit key from a larger secret key, “PBKDF2 with HMACSHA256” must be used with 1 iteration and no salt.

Structure:
KEYEXCHANGE;<Number>;<Time>;<Sender>;<Classification>;<Acknowledgement>;<MAC>;<Recipient>;<AlgorithmType>(M);<Phase>(M); <KeyLengthSharedSecret>(M*);<KeyLengthKEM>(M*);<Prime>(M*);<Natural number>(M*);<InitialisationVector>(M*);<Public variable\key>(M*)

* Whether it is required depends on the phase and the type of algorithm

| AlgorithmType | 0 | DH (Diffie-Hellman-Merkle 1024/2048 bit length, NIST SP 800-56A/B & 90A/B) |
| --- | --- | --- |
|  | 1 | ECDH (Diffie-Hellman-Merkle with Curve25519 / X25519, 1024/2048/4096 bit length) |
|  | 2 | Kyber-512 |
|  | 3 | Kyber-768 |
|  | 4 | Kyber-1024 |
|  | 5 | FrodoKEM-640 (AES) |
|  | 6 | FrodoKEM-976 (AES) |
|  | 7 | FrodoKEM-1344 (AES) |
| Phase | 0,1,2 | Defines the current step in the key exchange process |
| KeyLengthSharedSecret | 128, 256 | Bit length of the shared secret key (Phase 0) |
| KeyLengthKEM | 1024, 2048, 4096 | Bit length of the key used for DH or KEM process (Phase 0) |
| Prime (p) | HexString | Publicly known prime number (> 3000 bits / 375 byte) (Phase 0, DH only) |
| Natural number (g) | HexString | Publicly known natural number smaller than p (Phase 0, DH only) |
| Initialisation vector (IV) | HexString | Initialisation vector (IV) (For AES CFB/CTR encryption only) |
| Public variable/key | BASE64 | The public variable/key of the sender |

Sample 1: KEYEXCHANGE;0;0191C643A8AF;89AD;U;;;FE2A;0;128;1024;7FFFFFFF;822460DE

Sample 2: KEYEXCHANGE;0;0191C643A8AF;FE2A;U;;;89AD;1;128;2048;;6E6026EFF9D9EBEB9D4A973CB5C287DBD77D75EDDD2

DH-, ECDH and Kyber/FrodoKEM sequences:

:

- SEDAP-Express JSON-Schema

The JSON schema was intentionally kept very simple. The JSON message contains only a list of one or more SEDAP-Express messages in their original (JSON-compatible) format. This means that the same message classes could be used for parsing and generating. With GET request the client can retrieve messages. On the other side with POST the client can send messages to the server. For simplicity there is only one endpoint at all. You can mix different messages in a single POST request. Updates for existing messages, e.g. contacts, can be sent to the server with POST or alternatively with PUT.

Endpoint:

http://<ip>:<port>/SEDAPEXPRESS

Schema:

{

"messages":[

{

"message":""

}

]

}

Sample GET request:

GET /SEDAPEXPRESS HTTP/1.1
Host: sample.host
Accept: application/json

Sample GET answer:

HTTP/1.1 200 OK

Content-Type: application/json

Content-Length: 380

{

"messages":[

{

"message":"CONTACT;60;0191C643A8AF;66A3;S;TRUE;102;TRUE;53.32;8.11",

"message":"METEO;AC;0191C643A8AF;74BE;U;;15.4;15.5;;;10.2;72;20.3;;55;1005;25;;;2500;33",

"message":"TEXT;D6;0191C643A8AF;324E;S;;3;This is a chat message!;E4F1",

"message":"GRAPHIC;79;0191C643A8AF;910E;U;;8;1;FF8000;Area A;10000;53.43;9.45"

}

]

}

Sample POST request:

POST /SEDAPEXPRESS HTTP/1.1
Host: sample.host
Accept: application/json

{

"messages":[

{

"message":" OWNUNIT;5E;0191C643A8AF;66A3;R;TRUE;;42.32;-123.11;10000;50.23;297;;;33.3;-0.15;sfapmf---------"

"message":"TEXT;AE;0191C643A8AF;374E;S;;3;This is a chat message!;E4F1"

}

]

}

Sample POST answer:

HTTP/1.1 200 OK

Content-Type: application/json

{"success":"true"}

- SEDAP-Express Protobuf-Definition

The Google™ Protocol buffer definition is kept very simple, too. As with JSON messages, the same classes could be used to parse or generate the actual content.

Definiton:

syntax = "proto3";

message SomeMessage {

message Messages {

string message = 1;

}

repeated Messages messages = 1;

}

- Contact

Please report any issues, suggestions, additions or whatever you have in mind with SEDAP-Express via GitHub, by email or by phone.

| GitHub: | https://github.com/UNIITY-Team |
| --- | --- |
| Internet: | https://www.linkedin.com/in/volker-voss |